CTE ARCHITECTURE & CONSTRUCTION · GRADES 9–12 · EARLY IN THE PROGRAM

Site to Scene: A Massing Study

Students respond to a client program with a quick massing model, place it on a real site at true scale, and publish it to ArcGIS Online as a 3D web scene for review.

| Course level | Introductory architecture or design drafting (first semester) |
| --- | --- |
| Time | Two 50-minute class periods |
| Grouping | Individual, with a peer review partner |
| Materials | GeoBlocks at tbaker.com/geoblocks ArcGIS Online account for each student (see Teacher setup) Student brief and rubric (pages 3–4) |
| Big idea | Architects test early ideas as simple masses on a real site before drawing any details. Scale, orientation, and context shape every design decision. |

Standards

| Code | Standard |
| --- | --- |
| CCTC AC01 | Use vocabulary, symbols, and formulas commonly used in design and construction. |
| CCTC ACDP01 | Justify design solutions through the use of research documentation and analysis of data. |
| CCTC ACDP05 | Identify the diversity of needs, values, and social patterns in project design, including accessibility standards. |
| CCTC ACDP06 | Apply the techniques and skills of modern drafting, design, engineering, and construction to projects. |
| CCTC ACDP07 | Employ appropriate representational media to communicate concepts and design. |
| C3 D2.Geo.1.9-12 | Use geospatial and related technologies to create maps to display and explain the spatial patterns of cultural and environmental characteristics. |
| ISTE 1.6 | Creative Communicator: students communicate clearly and express themselves creatively for a variety of purposes using the platforms, tools, styles, formats, and digital media appropriate to their goals. |

CCTC: Common Career Technical Core, Architecture & Construction Career Cluster, Design/Pre-Construction pathway (Advance CTE). States that have adopted the modernized Career Clusters framework can crosswalk these statements to their current course standards.

Learning targets

- I can translate a client program into a massing model at a stated scale.

- I can place a design on a real site using latitude, longitude, and orientation, and evaluate it in context.

- I can publish a 3D web scene to ArcGIS Online and share it for professional review.

- I can justify my design decisions using site evidence and correct design vocabulary.

Vocabulary

| Term | Meaning in this project |
| --- | --- |
| Program | The client's list of required spaces, sizes, and uses. |
| Massing | The overall 3D shape and size of a building, before details like windows or materials. |
| Footprint | The area of ground a building covers, in square meters. |
| Setback | The minimum distance a building must stay from the property line. |
| Orientation | The direction a building faces, measured in degrees clockwise from north. |
| Floor-to-floor height | The vertical distance from one floor to the next. Here, 3 blocks = 3.6 m. |
| Web scene | A 3D map in ArcGIS Online that others can open, explore, and comment on. |

Teacher setup (before day 1)

- Accounts: Publishing creates a hosted 3D layer, so students need an ArcGIS Online Creator user type with publishing privileges. Check under Organization → Members.

- Sign-in method: GeoBlocks signs in with an ArcGIS username and password. Accounts that only sign in through a school Google, Microsoft, or other single sign-on button cannot sign in from GeoBlocks. Use built-in ArcGIS accounts for this project.

- Class group: Create an ArcGIS Online group (for example, “Arch 1 Massing Studies”) and add your students so you can find every submission in one place.

- What gets saved: Each publish adds two items to the student's content: a web scene and a read-only hosted feature layer of the blocks. Both are small.

- Test once: Publish one model yourself with a student-type account before class to confirm your organization's settings allow it.

Teacher steps

Day 1 · Program to massing (50 minutes)

- Hook (5 min): Show two photos of real massing models from a local firm or a well-known building. Ask what the model tells a client, and what it leaves out.

- Brief (10 min): Read the client program on the student page. Have students calculate the footprint and height targets in studs and blocks before building (Part A).

- Set up the site (5 min): In Map settings, students choose Preset locations → Prairie (Flint Hills, Kansas) and set Scale to 1 meter per stud. Leave rotation at 0 for now.

- Mass the building (25 min): Students build the hall and lookout tower as simple solid masses. Encourage two or three quick options rather than one perfect one.

- Save (5 min): File → Save file downloads a .geoblocks file. Students keep it for day 2.

Day 2 · Site, publish, and review (50 minutes)

- Site check (10 min): Students switch to 3D Map to see the terrain, roads, and views. They adjust Rotation from north so the entry faces the approach road and the lookout faces the best view. Record the angle.

- Publish (10 min): Export → ArcGIS Online → Sign in, then Save GeoBlock creation to ArcGIS. Name it Lastname_VisitorCenter_Massing, choose My organization, and save. Copy the web scene link.

- Share (5 min): In ArcGIS Online, share the web scene with the class group. Paste the link on the student page.

- Peer review (15 min): Partners open each other's scene in Scene Viewer. They use the Daylight tool to check shadows at 9 a.m. and 3 p.m. and leave one strength and one question.

- Design rationale (10 min): Students write a 150-word rationale using at least four vocabulary terms (Part C).

Teaching notes Scale check: at 1 stud = 1 m, a block is 1.2 m tall, so a typical 3.6 m floor-to-floor height equals 3 blocks. A 4 × 4 m tower needs a 4 × 4 stud footprint. Accessibility: require the main entry at ground level (level 0) with no steps. Discuss what the model cannot show, such as ramps, door widths, and clearances. Extend: have students export a GLB model (Export → Export 3D model) and bring it into ArcGIS Pro, SketchUp, or Blender for the next stage of design development.

STUDENT BRIEF

Site to Scene: A Massing Study

Name: ______________________________   Review partner: ______________________________

Open GeoBlocks: https://tbaker.com/geoblocks

Client program: Flint Hills Visitor Center The county wants a small visitor center on a prairie site in the Flint Hills of Kansas. Main hall: one story, between 150 m² and 220 m² in footprint. Lookout tower: at least two stories tall (7.2 m or more), with a footprint no larger than 36 m². Total footprint: no more than 300 m². Setback: keep the building at least 5 m from every edge of the GeoBlocks mat (the property line). Entry: at ground level with no steps, facing the approach road. Scale: 1 stud = 1 m. One block = 1.2 m tall. One story (3.6 m) = 3 blocks.

Part A · Translate the program

| Question | Answer with work |
| --- | --- |
| Hall footprint range in stud squares |  |
| One possible hall size (length × width in studs) |  |
| Tower height in blocks (two stories minimum) |  |
| Largest tower footprint (studs × studs) |  |
| Setback from the mat edge, in studs |  |

Part B · Site and publishing record

| Item | Your value |
| --- | --- |
| Site latitude, longitude |  |
| Rotation from north (degrees) and why |  |
| Final footprint (m²) and tallest point (m) | (from Creation size) |
| Web scene title |  |
| Web scene link |  |

Part C · Design rationale (about 150 words)

Explain how your massing responds to the program and the site. Use at least four of these terms: program, massing, footprint, setback, orientation, scale, accessibility, web scene.

Rubric

| Criterion | 4 · Exceeds | 3 · Meets | 2 · Approaching | 1 · Beginning |
| --- | --- | --- | --- | --- |
| Program fit (ACDP01) | All program items met, with a clear reason for each size | All program items met | One item missed | Two or more items missed |
| Scale and site (AC01, D2.Geo.1) | Correct scale; orientation and setback justified with site evidence | Correct scale, setback, and orientation | One scale or setback error | Scale not applied |
| Accessibility (ACDP05) | Ground-level entry plus notes on what else is needed | Ground-level entry facing the road | Entry present but not at ground level | No clear entry |
| Published scene (ACDP07, ISTE 1.6) | Published, shared, titled correctly, peer feedback answered | Published and shared with the class group | Published but not shared or titled correctly | Not published |
| Rationale (AC01) | Clear, specific, and uses 5+ terms correctly | Uses 4 terms correctly | Uses 2–3 terms | Fewer than 2 terms |