STEM · GRADE 7 MATH AND ENGINEERING

Pavilion by the Numbers

Students design a picnic pavilion for a town park, use a scale to predict its real size, build it in GeoBlocks, and test their design against the criteria.

| Grade level | Grade 7 (adaptable for grades 6–8) |
| --- | --- |
| Time | One 55–60 minute class period |
| Grouping | Teams of two or three |
| Materials | GeoBlocks at tbaker.com/geoblocks Design sheet (pages 3–4), one per student Calculators |
| Big idea | A scale connects a model to the real world. Lengths grow by the scale factor, but areas grow by the scale factor squared. |

Standards

| Code | Standard |
| --- | --- |
| NGSS MS-ETS1-1 | Define the criteria and constraints of a design problem with sufficient precision to ensure a successful solution, taking into account relevant scientific principles and potential impacts on people and the natural environment that may limit possible solutions. |
| NGSS MS-ETS1-2 | Evaluate competing design solutions using a systematic process to determine how well they meet the criteria and constraints of the problem. |
| CCSS.MATH 7.G.A.1 | Solve problems involving scale drawings of geometric figures, including computing actual lengths and areas from a scale drawing and reproducing a scale drawing at a different scale. |
| CCSS.MATH 7.RP.A.2 | Recognize and represent proportional relationships between quantities. |
| CCSS Math Practice 4 | Model with mathematics. |

Learning targets

- I can use a scale to change model measurements into real-world lengths and areas.

- I can explain why area changes by the scale factor squared.

- I can check a design against criteria and constraints and suggest an improvement.

The design brief

Client: Riverside Town Parks Department We need a picnic pavilion for our new park. Your team will design it in GeoBlocks. Scale: 1 stud = 2 meters. (A stud is the width of a 1 × 1 block.) Each block is 1.2 studs tall. Criteria (must have): the roof covers at least 96 m², and people can stand under the roof, so the bottom of the roof is at least 4.8 m above the ground. Constraints (limits): the footprint is no bigger than 24 m × 16 m, and the budget allows 60 blocks or fewer.

Teacher steps

1. Launch (8 minutes)

- Read the brief together. Have students sort the four requirements into criteria (what success looks like) and constraints (limits).

- Demonstrate Map settings → Scale: set the length of one stud to 2 meters. Open Creation size to show that GeoBlocks reports real sizes.

2. Predict before building (12 minutes)

Teams complete Part A of the design sheet: they convert the brief into studs and blocks before touching the computer. Circulate and ask, “How did you know to divide by 2?”

3. Build and test (20 minutes)

- Teams set the scale to 2 m, then build their pavilion. Posts can be 1 × 1 or 2 × 2 blocks, and the roof sits on top of the posts.

- They open Creation size to check the real footprint and height, and count blocks with the Bricks number.

- They fill in Part B and fix anything that does not meet the brief.

- Optional: Export → Export image for a design portfolio.

4. Change the scale (8 minutes)

Teams change the scale to 1 stud = 1 meter without changing a single block and complete Part C. This is where students discover that area shrinks to one-fourth, not one-half.

5. Evaluate designs (7 minutes)

Teams swap screens with another team and score that design in Part D. Close by asking: “Which design best balanced shade and budget? What trade-offs did they make?”

Answer key

| Question | Answer |
| --- | --- |
| Largest footprint in studs (A1) | 24 m ÷ 2 = 12 studs by 16 m ÷ 2 = 8 studs |
| Height of one block (A2) | 1.2 studs × 2 m = 2.4 m |
| Blocks of posts under the roof (A3) | 4.8 m ÷ 2.4 m = 2 blocks |
| Area of one stud square (A4) | 2 m × 2 m = 4 m² |
| Roof squares for 96 m² (A5) | 96 ÷ 4 = 24 stud squares, for example a 6 × 4 roof |
| At 1 stud = 1 m (Part C) | Every length halves (scale factor ½). Area becomes ¼: a 6 × 4 roof covers 24 m² instead of 96 m². Post height drops from 4.8 m to 2.4 m. |

Teaching notes Sample design: four 2 × 2 corner posts, two blocks tall (8 blocks), topped by a roof of 2 × 4 blocks covering 8 × 6 studs (6 blocks). That is 14 blocks, a 16 m × 12 m footprint, and 192 m² of shade. Watch for: students using 2 m for block height. Support: model one row of Part A. Extend: find the cheapest design that meets every criterion, or 3D print it (Export → Export 3D model → STL).

DESIGN SHEET

Pavilion by the Numbers

Name: ______________________________   Team: ______________________________

Open GeoBlocks: https://tbaker.com/geoblocks

Scale: 1 stud = 2 m. A block is 1.2 studs tall. Criteria: roof covers at least 96 m²; roof bottom at least 4.8 m high. Constraints: footprint no bigger than 24 m × 16 m; 60 blocks or fewer.

Part A · Predict before you build

| Question | Show your work |
| --- | --- |
| A1. What is the largest footprint allowed, in studs? |  |
| A2. How tall is one block, in meters? |  |
| A3. How many blocks tall must the posts be so the roof is at least 4.8 m up? |  |
| A4. What real area does one stud square cover? |  |
| A5. How many stud squares must the roof cover to reach 96 m²? |  |

Part B · Build and test

Set Map settings → Scale to 2 meters and build. Open Creation size to check your design.

| Requirement | Our design | Met? (yes / no) |
| --- | --- | --- |
| Footprint ≤ 24 m × 16 m |  |  |
| Roof bottom ≥ 4.8 m |  |  |
| Roof covers ≥ 96 m² |  |  |
| 60 blocks or fewer |  |  |

Part C · Change the scale

Change the scale to 1 stud = 1 m. Do not move any blocks.

C1. New footprint: __________ m × __________ m     New roof area: __________ m²

C2. The lengths were multiplied by ______. The area was multiplied by ______. Why are these two numbers different?

Part D · Evaluate another team's design

Score each requirement 0 (not met), 1 (barely met), or 2 (met well).

| Requirement | Score (0–2) | Evidence |
| --- | --- | --- |
| Shade: roof covers ≥ 96 m² |  |  |
| Headroom: roof ≥ 4.8 m high |  |  |
| Fits the footprint |  |  |
| Within budget (≤ 60 blocks) |  |  |
| Total (out of 8) |  |  |

One improvement we suggest for their design: